Tourism District Landscape Maintenance for January

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger.

- Mulch beds using double-ground dyed brown mulch. (Mulching depths depend on the situation. If an area has been mulched regularly only 1-2 inches may be needed to improve color. If an area hasn’t been mulched in recent seasons or bare soil is showing it is necessary to mulch to a 2–3-inch depth.)

Park Amenities Maintenance

- ‘Spring’ cleaning of elevators, plazas, and bridge.

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

- Apply ice-melt to icy patches on sidewalks during freezing temperatures.

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for aphids as needed.

- Spray for fungal diseases as needed.

- Apply liquid soluble fertilizer per label as needed

- Take/process soil samples.

- Hand water as needed.

Perennial Maintenance

- Cut back liriope towards the end of month.

- Cut back dormant perennials that have been damaged by ice or wind, including ornamental grasses.

Ornamental Maintenance

- Inspect plants for scale, aphids, and other pests.

- Inspect plants for broken or damaged branches due to ice or wind. Prune as necessary.

- Apply water during unusually dry periods.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Lawn Maintenance

- Warm-season grasses

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Take/process soil samples.

  - Monitor turf for pest invasion

- Cool-season grasses

  - Apply fertilizer based on soil samples

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Take soil samples.

  - Monitor turf for pest invasion

Tourism District Landscape Maintenance for February

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed.

- Spray goose repellant to targeted turf areas

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Apply pre-emergent herbicide. (For beds close to river, Dimension is sprayed to reduce risk of runoff contamination to river. Other beds are treated with granular Snapshot.)

- Mulch beds using double-ground dyed brown mulch. (Mulching depths depend on the situation. If an area has been mulched regularly only 1-2 inches may be needed to improve color. If an area hasn’t been mulched in recent seasons or bare soil is showing it is necessary to mulch to a 2–3-inch depth.)

Park Amenities Maintenance

- ‘Spring’ cleaning of elevators, plazas, and bridge.

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

- Apply ice-melt to icy patches on sidewalks during freezing temperatures.

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for aphids as needed.

- Spray for fungal diseases as needed.

- Apply liquid soluble fertilizer per label as needed

- Hand water as needed.

Perennial Maintenance

- Cut back any remaining dormant perennial foliage. (Cut ornamental grasses to within 6” of the ground. Cut most perennials with woody stems to within 6” of the ground. Perennials with herbaceous stems and leaves should be cut to within 2” of the ground.)

- As spring flowering bulbs start to flower and decline, deadhead spent blooms and remove spent blooms ONLY. DO NOT REMOVE FOLIAGE.

Ornamental Maintenance

- Inspect plants for scale, aphids, and other pests.

- Inspect plants for broken or damaged branches due to ice or wind. Prune as necessary.

- Apply water during unusually dry periods.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Lawn Maintenance

- Warm-season grasses

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Take/process soil samples.

  - Apply pre-emergent

  - Monitor turf for pest invasion

- Cool-season grasses

  - Apply fertilizer based on soil samples

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Take soil samples.

  - Apply pre-emergent

  - Monitor turf for pest invasion

Tourism District Landscape Maintenance for March

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed.

- Pressure wash Liberty Bridge

- Spray goose repellant to targeted turf areas (if not completed in February)

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Mulch beds using double-ground dyed brown mulch. (Mulching depths depend on the situation. If an area has been mulched regularly only 1-2 inches may be needed to improve color. If an area hasn’t been mulched in recent seasons or bare soil is showing it is necessary to mulch to a 2–3-inch depth.)

Park Amenities Maintenance

- ‘Spring’ cleaning of elevators, plazas, and bridge.

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

- Prepare flower yard for deliveries

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for aphids as needed.

- Spray for fungal diseases as needed.

- Apply liquid soluble fertilizer per label as needed

- Hand water as needed.

Perennial Maintenance

- Apply fertilizer per plant species (usually one handful).  Apply fertilizer in a ‘ring’ around each individual plant when possible. For groundcovers or tightly-spaced plants, broadcast fertilizer evenly over area. Never apply fertilizer directly on the base, crown, or leaves of the plant.

Ornamental Maintenance

- Apply fertilizer per plant species.  Apply fertilizer in a ‘ring’ around each individual plant. Never apply fertilizer directly on the base, crown, or leaves of the plant.

- Prune out any damaged or dead wood.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Lawn Maintenance

- Cool-season grasses

  - Spray broadleaf weeds with Speed Zone Southern (or another applicable herbicide) per label.

  - Core aerate lawn areas.

  - Overseed fescue laws @ 6 lbs. of seed per 1000 sq. ft. Apply high nitrogen fertilizer at labeled rate.

  - Apply pelletized dolomitic lime if soil test requires it.

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

Tourism District Landscape Maintenance for April

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed

- Spray goose repellant to targeted turf areas

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Touch up mulch beds as needed

Park Amenities Maintenance

- ‘Spring’ cleaning of elevators, plazas, and bridge.

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Pull spent tulips and discard them.

- At months end, begin removing annuals for seasonal change out.

- Hand water as needed.

Perennials

- Mid to late month is a good time to replant perennial plant material.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Irrigation

- Turn irrigation systems on for growing season.

- Test systems to ensure proper coverage of areas.

- Make any necessary repairs.

- Program system to apply 1 inch of water per week to all bed and lawn areas.

Lawn Maintenance

- Warm-season grasses

  - Pull Aerway aerifier.

  - Fertilize if soil sample requires it.

  - Spray broadleaf weeds with Speed Zone Southern per label.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

Tourism District Landscape Maintenance for May

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed

- Spray goose repellant to targeted turf areas (if not completed in April)

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Touch up mulch beds as needed

Park Amenities Maintenance

- Cleaning of elevators, plazas, and bridge as needed

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

Annual Maintenance

- Plant early summer annuals. Prepare beds by tilling 10-10-10 fertilizer into the top 6-8 inches of soil. Rake the beds smooth and top dress with Osmocote. Plant annuals and smooth soil between plants. Apply compost at  ½” to cover soil surface. Water plants in until soil is ‘soupy’.

- Add lime if soil sample requires it.

- Hand water as needed.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Lawn Maintenance

- Warm-season grasses

  - Core aerate lawn areas.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

Tourism District Landscape Maintenance for June

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed.

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Touch up mulch beds as needed

Park Amenities Maintenance

- Cleaning of elevators, plazas, and bridge as needed

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for insects as needed.

- Spray for fungal diseases as needed.

- Apply liquid fertilizer if needed

- Deadhead flowers as needed. Pinch begonias back by half.

- Hand water as needed.

Perennial Maintenance

- Deadhead flowers as needed.

- Prune to keep perennials looking neat and in proportion.

- Spray for insect s or fungal diseases as necessary.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water. Adjust irrigation times as necessary. Also check irrigation patterns to ensure irrigation is covering all areas.

Lawn Maintenance

- Warm-season grasses

  - Core aerate lawn areas.

  - Apply fertilizer based on soil samples.

  - Top-dress lawn areas using sand.

  - Mow, weed eat, and edge lawn areas at a height of 1 inch. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

- Cool-season grasses

  - Apply 5-10-31 + 10% Iron @ 1 bag per 15,000 sq. ft. per LESCO.

  - For high-profile areas, apply preventative application of fungicide to lawn areas.

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

Tourism District Landscape Maintenance for July

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed

- Spray goose repellant to targeted turf areas

- Pressure wash Liberty Bridge after July 4th

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Apply pre-emergent herbicide. (For beds close to river, Dimension is sprayed to reduce risk of runoff contamination to river. Other beds are treated with granular Snapshot.)

- Touch up mulch beds as needed

Park Amenities Maintenance

- Cleaning of elevators, plazas, and bridge as needed

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for insects as needed.

- Spray for fungal diseases as needed.

- Deadhead flowers as needed. Pinch begonias back by half.

- Hand water as needed.

Perennial Maintenance

- Deadhead flowers as needed.

- Prune to keep perennials looking neat and in proportion.

- Spray for insects or fungal diseases as necessary.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water. Adjust irrigation times as necessary. Also check irrigation patterns to ensure irrigation is covering all areas.

Lawn Maintenance

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 1 inch. Blow grass clippings from paved surfaces.

  - Pull Aerway aerifier.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion (ex. Army worms)

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion (ex. Army worms)

Tourism District Landscape Maintenance for August

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed

- Spray goose repellant to targeted turf areas (if not completed in July)

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Touch up mulch beds as needed

Park Amenities Maintenance

- Cleaning of elevators, plazas, and bridge as needed

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for insects as needed.

- Spray for fungal diseases as needed.

- Apply liquid fertilizer as needed

- Deadhead flowers as needed.  Do not pinch begonias unless absolutely necessary.

- Remove ‘tired’ annuals such as petunias and cleome and replace with marigolds to freshen up beds.

- Hand water as needed.

Perennial Maintenance

- Deadhead flowers as needed.

- Prune to keep perennials looking neat and in proportion.

- Spray for insects or fungal diseases as necessary.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water. Adjust irrigation times as necessary. Also check irrigation patterns to ensure irrigation is covering all areas.

Lawn Maintenance

- Warm-season grasses

  - Apply 28-5-12 50% Polyplus N + 3% Iron @ 1 bag per 14,000 sq. ft. per LESCO.

  - Mow, weed eat, and edge lawn areas at a height of 1 inch. Blow grass clippings from paved surfaces.

  - Pull Aerway aerifier.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary

  - Monitor turf for pest invasion (ex. Army worms)

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion (ex. Army worms)

Tourism District Landscape Maintenance for September

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed.

- Spray goose repellant to targeted turf areas

General Bed Maintenance

- Pull and spray weeds.

- Remove trash and cigarette butts from beds.

- Touch up mulch beds as needed

Park Amenities Maintenance

- Cleaning of elevators, plazas, and bridge as needed

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls)

- Prepare flower yard for deliveries

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for insects as needed.

- Spray for fungal diseases as needed.

- Deadhead flowers as needed.

- Hand water as needed.

Perennial Maintenance

- Deadhead flowers as needed.

- Prune to keep perennials looking neat and in proportion.

- Spray for insects or fungal diseases as necessary.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water. Adjust irrigation times as necessary. Also check irrigation patterns to ensure irrigation is covering all areas.

Lawn Maintenance

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 1 inch. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion (ex. Army worms)

- Cool-season grasses

  - Overseed fescue @ a rate of 8lbs. per 1000 sq. ft. Apply 18-24-12 starter fertilizer at the labeled rate.

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion (ex. Army worms)

Tourism District Landscape Maintenance for October

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed

- Spray goose repellant to targeted turf areas (if not completed in September)

General Bed Maintenance

- Remove trash and cigarette butts from beds.

- Blow, rake, and remove leaves from beds.

- Touch up mulch beds as needed

Annual Maintenance

- Plant winter annuals. Prepare beds by tilling 10-10-10 fertilizer into the top 6-8 inches of soil. Rake the beds smooth and top dress with Osmocote. Plant annuals and smooth soil between plants. Apply compost ½” to cover soil surface. Water plants in until soil is ‘soupy’.

- Hand water as needed.

Perennial Maintenance

- Prune any unsightly perennials to neaten their appearance.

- Spray for insects or fungal diseases as necessary.

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Irrigation

- Start adjusting irrigation times back to prevent areas from being over-watered. Newly planted annuals and over-seeded lawns will need continued irrigation.

Lawn Maintenance

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 2 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

Planting

- Plant replacement trees and shrubs as needed.

Tourism District Landscape Maintenance for November

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep and litter pick daily

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed.

- Spray goose repellant to targeted turf areas (if not completed in September or October)

General Bed Maintenance

- Remove trash and cigarette butts from beds. Touch up mulch beds as needed

- Blow, rake, and remove leaves from beds.

Annual Maintenance

- Late November start planting spring-flowering bulbs. Plant tulips 6-inches deep on 4-inch spacing. Plant daffodils 6-inches deep on 6-inch spacing. Hand water annuals as needed

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Lawn Maintenance

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 2 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

Planting

- Plant replacement trees and shrubs as needed

Tourism District Landscape Maintenance for December

General Park Maintenance

- Check restrooms/elevators at Falls Park and River Place daily.

- Change trash bags in trash receptacles at Falls Park and River Place, Pedrick’s Garden, Cancer Survivor’s Park and Linky Stone Children’s Garden

- Inspect and remove spider webs

- River sweep daily.

- Litter pick park daily.

- Check dog-waste bag supply daily.

- Straighten tables at Tate Plaza daily.

- Blow walkways 3 days per week.

- Inspect light bulbs twice monthly.

- Hand water or irrigate as needed.

General Bed Maintenance

- Remove trash and cigarette butts from beds.

- Blow, rake, and remove leaves from beds.

- Touch up mulch beds as needed

Annual Maintenance

- Finish planting spring-flowering bulbs, hand water annuals as needed

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trunks and sprayed around the base periodically

Lawn Maintenance

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 2”. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Monitor turf for pest invasion

Planting

- Plant t trees and shrubs as needed