2/14/2023

Unity Park Splash Pad Maintenance SOP

Working Document

# Splash Pad Maintenance

## April 15 – April 30

| - Pressure wash pad | - Clean jets/main return drains |
| --- | --- |
| - Clean Vaults | - Install Probes |
| - Clean Chlorinators | - Check Programming issues |
| - Fill Vaults | - De-water pump |
| - Fill lines in System | - Backwash |
| - Prepare for Opening | - Fill line |
| - Order new logbook | - Bring system out of winterization UV-System (Manual) |
| - Order chemicals needed |  |

## May 1 – May 31

| - Daily trash check | - Run pool skimmer around in vaults |
| --- | --- |
| - M-W-F Clean all jets; check pipe for adequate flow; take jets apart | - Every 3 weeks – Clean Probes (not out of water more than 2 hours – use toothbrush and toothpaste) |
| - T-TH-S Clean all return drains | - Check levels 2x Daily – Chlorine, PH, CYA Alkalinity 1 week |
| - SUN Clean chlorinators; check for blockage | - UV System – Check for issue lights |
| - Skimmer baskets – clean daily in pit/vault |  |

## June 1 – June 30

| - Daily trash check | - Run pool skimmer around in vaults |
| --- | --- |
| - M-W-F Clean all jets; check pipe for adequate flow; take jets apart | - Every 3 weeks – Clean Probes (not out of water more than 2 hours – use toothbrush and toothpaste) |
| - T-TH-S Clean all return drains | - Check levels 2x Daily – Chlorine, PH, CYA Alkalinity 1 week |
| - SUN Clean chlorinators; check for blockage | - UV System – Check for issue lights |
| - Skimmer baskets – clean daily in pit/vault |  |
| - SUN Clean chlorinators; check for blockage |  |

## July 1 – July 31

| - Daily trash check | - Run pool skimmer around in vaults |
| --- | --- |
| - M-W-F Clean all jets; check pipe for adequate flow; take jets apart | - Every 3 weeks – Clean Probes (not out of water more than 2 hours – use toothbrush and toothpaste) |
| - T-TH-S Clean all return drains | - Check levels 2x Daily – Chlorine, PH, CYA Alkalinity 1 week |
| - SUN Clean chlorinators; check for blockage | - UV System – Check for issue lights Add chemicals as needed: - Alkalinity 80-120 ppm (100) - CYA 30-100 ppm (50) - PH 7.2-7.8 ppm (7.5) - Chlorine 3.0-5.0 ppm (5.0) - Holidays are busy |
| - Skimmer baskets – clean daily in pit/vault |  |
| - SUN Clean chlorinators; check for blockage |  |

## August 1 – August 31

| - Daily trash check | - Run pool skimmer around in vaults |
| --- | --- |
| - M-W-F Clean all jets; check pipe for adequate flow; take jets apart | - Every 3 weeks – Clean Probes (not out of water more than 2 hours – use toothbrush and toothpaste) |
| - T-TH-S Clean all return drains | - Check levels 2x Daily – Chlorine, PH, CYA Alkalinity 1 week |
| - SUN Clean chlorinators; check for blockage | - UV System – Check for issue lights |
| - Skimmer baskets – clean daily in pit/vault |  |
| - SUN Clean chlorinators; check for blockage |  |

## September 1 – September 30

| - Daily trash check | - Run pool skimmer around in vaults |
| --- | --- |
| - M-W-F Clean all jets; check pipe for adequate flow; take jets apart | - Every 3 weeks – Clean Probes (not out of water more than 2 hours – use toothbrush and toothpaste) |
| - T-TH-S Clean all return drains | - Check levels 2x Daily – Chlorine, PH, CYA Alkalinity 1 week |
| - SUN Clean chlorinators; check for blockage | - UV System – Check for issue lights |
| - Skimmer baskets – clean daily in pit/vault | - Check call box operation and address |
| - SUN Clean chlorinators; check for blockage |  |

## October 1 – October 15

| - Daily trash check | - Skimmer baskets – clean daily in pit/vault |
| --- | --- |
| - M-W-F Clean all jets; check pipe for adequate flow; take jets apart | - SUN Clean chlorinators; check for blockage |
| - T-TH-S Clean all return drains | - Run pool skimmer around in vaults |
| - SUN Clean chlorinators; check for blockage |  |

# Closing procedure

| - Turn off main disconnect | - Clean Chlorinators |
| --- | --- |
| - Drain vaults; de-water pump | - Pump out Vaults (ShopVac) |
| - Drain all lines, filter, jet pumps | - Pressure wash pad/drains |
| - Drain 10 inch line in Pit (Plug bottom center) | - Walk through double check clean/off/empty |
| - Drain Pool filters (Pull plug on line to filters) |  |
| - Store Probes (cup of water) |  |