Neighborhood Parks Landscape Maintenance

for January

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

- Mulch beds using double-ground dyed brown mulch. (Mulching depths depend on the situation. If an area has been mulched regularly only 1-2 inches may be needed to improve color. If an area hasn’t been mulched in recent seasons or bare soil is showing it is necessary to mulch to a 2–3-inch depth.)

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for aphids as needed.

- Spray for fungal diseases as needed

- Apply liquid soluble fertilizer per label

- Take/process soil samples if issues present

- Hand water or irrigate as needed

Perennial Maintenance

- Cut back liriope towards the end of month

- Cut back dormant perennials that have been damaged by ice or wind, including ornamental grasses

Ornamental Maintenance

- Inspect plants for scale, aphids, and other pests

- Inspect plants for broken or damaged branches due to ice or wind. Prune as necessary

- Apply water during unusually dry periods

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water.

- Adjust irrigation times as necessary.

- Check irrigation patterns to ensure irrigation is covering all areas.

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Apply fertilizer based on soil samples.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

  - Take soil samples.

Neighborhood Parks Landscape Maintenance

for February

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

- Mulch beds using double-ground dyed brown mulch. (Mulching depths depend on the situation. If an area has been mulched regularly only 1-2 inches may be needed to improve color. If an area hasn’t been mulched in recent seasons or bare soil is showing it is necessary to mulch to a 2–3-inch depth.)

- Apply pre-emergent herbicide (For beds close to river, Dimension is sprayed to reduce risk of runoff contamination to river. Other beds are treated with granular Snapshot.)

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for aphids as needed.

- Spray for fungal diseases as needed.

- Apply liquid soluble fertilizer per label.

- Hand water or irrigate as needed

Perennial Maintenance

- Cut back any remaining dormant perennial foliage. (Cut ornamental grasses to within 6” of the ground. Cut most perennials with woody stems to within 6” of the ground. Perennials with herbaceous stems and leaves should be cut to within 2” of the ground.)

- As spring flowering bulbs start to flower and decline, deadhead spent blooms and apply 10-10-10 at 1 lb. per 100 sq. ft. Remove spent blooms only. DO NOT REMOVE FOLIAGE.

Shrub Maintenance

- Hard prune shrubs per species in late February

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water.

- Adjust irrigation times as necessary.

- Check irrigation patterns to ensure irrigation is covering all areas.

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

Neighborhood Parks Landscape Maintenance

for March

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

- Mulch beds using double-ground dyed brown mulch. (Mulching depths depend on the situation. If an area has been mulched regularly only 1-2 inches may be needed to improve color. If an area hasn’t been mulched in recent seasons or bare soil is showing it is necessary to mulch to a 2–3-inch depth.)

- Apply pre-emergent herbicide (For beds close to river, Dimension is sprayed to reduce risk of runoff contamination to river. Other beds are treated with granular Snapshot.)

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for aphids as needed.

- Spray for fungal diseases as needed.

- Apply liquid soluble fertilizer per label.

- Hand water or irrigate as needed

Perennial Maintenance

- Apply fertilizer as needed

Ornamental Maintenance

- Apply fertilizer as needed. Apply fertilizer in a ‘ring’ around each individual plant. Never apply fertilizer directly on the base or crown of the plant.

- Prune out any damaged or dead wood.

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water.

- Adjust irrigation times as necessary.

- Check irrigation patterns to ensure irrigation is covering all areas.

- Mark irrigation heads prior to aeration

Lawn Maintenance

- Mowing maintenance should consist of: mowing, weed eating, edging, blowing pathway

- Cool-season grasses

  - Spray broadleaf weeds with Speed Zone Southern (or another applicable herbicide) per label.

  - Core aerate lawn areas.

  - Overseed fescue lawns @ 6 lbs. of seed per 1000 sq. ft. Apply 18-24-12 starter fertilizer at labeled rate, if fescue is present

  - Apply pelletized dolomitic lime if soil test requires it.

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Neighborhood Parks Landscape Maintenance

for April

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues.

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

- Mulch beds using double-ground dyed brown mulch. (Mulching depths depend on the situation. If an area has been mulched regularly only 1-2 inches may be needed to improve color. If an area hasn’t been mulched in recent seasons or bare soil is showing it is necessary to mulch to a 2–3-inch depth.)

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Pull spent tulips and discard them.

- At months end, begin removing annuals for seasonal change out.

- Hand water or irrigate as needed

Perennials

- Maintenance as needed

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Turn irrigation systems on for growing season.

- Test systems to ensure proper coverage of areas.

- Make any necessary repairs.

- Program system to apply 1 inch of water per week to all bed and lawn areas.

- Mark irrigation heads prior to aeration

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Pull Aerway aerifier.

  - Fertilize if soil sample requires it.

  - Spray broadleaf weeds with Speed Zone Southern (or another applicable herbicide) per label.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Neighborhood Parks Landscape Maintenance

for May

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues.

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Plant spring annuals. Prepare beds by tilling 10-10-10 fertilizer into the top 6-8 inches of soil. Rake the beds smooth and top dress with Osmocote. Plant annuals and smooth soil between plants. Apply compost at ½” to cover soil surface. Water plants in until soil is ‘soupy’.

- Add lime if soil sample requires it.

- Hand water or irrigate as needed

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water.

- Adjust irrigation times as necessary.

- Check irrigation patterns to ensure irrigation is covering all areas.

- Mark irrigation heads prior to aeration

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Core aerate lawn areas.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Neighborhood Parks Landscape Maintenance

for June

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues.

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for insects as needed.

- Spray for fungal diseases as needed.

- Deadhead flowers as needed. Pinch begonias back by half.

- Hand water or irrigate as needed.

Perennial Maintenance

- Deadhead flowers as needed.

- Prune to keep perennials looking neat and in proportion.

- Spray for insects or fungal diseases as necessary.

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water.

- Adjust irrigation times as necessary.

- Check irrigation patterns to ensure irrigation is covering all areas.

- Mark irrigation heads prior to aeration

Lawn Maintenance

- Mowing maintenance should consist of: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Core aerate lawn areas.

  - Apply fertilizer based on soil samples.

  - Top-dress lawn areas using sand.

  - Mow, weed eat, and edge lawn areas at a height of 2.5-3.5 inches depending on the topography. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Apply 5-10-31 + 10% Iron @ 1 bag per 15,000 sq. ft. per LESCO, if present

  - For high-profile areas, apply preventative application of fungicide to lawn areas.

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Neighborhood Parks Landscape Maintenance

for July

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues.

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

- Apply pre-emergent herbicide. (For beds close to river, Dimension is sprayed to reduce risk of runoff contamination to river. Other beds are treated with granular Snapshot.)

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for insects as needed.

- Spray for fungal diseases as needed.

- Deadhead flowers as needed. Pinch begonias back by half.

- Hand water or irrigate as needed

Perennial Maintenance

- Deadhead flowers as needed.

- Prune to keep perennials looking neat and in proportion.

- Spray for insects or fungal diseases as needed.

- Apply fertilizer as needed to ensure healthy bloom and growth through fall.

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water.

- Adjust irrigation times as necessary.

- Check irrigation patterns to ensure irrigation is covering all areas.

- Mark irrigation heads prior to aeration

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 2.5-3.5 inches depending on topography. Blow grass clippings from paved surfaces.

  - Pull Aerway aerifier.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Neighborhood Parks Landscape Maintenance

for August

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues.

- Asses parks for tree/shrub/perennial replacement or additions, compile list

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

Pressure wash playground equipment as needed

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for insects as needed.

- Spray for fungal diseases as needed.

- Deadhead flowers as needed.  Do not pinch begonias unless absolutely necessary.

- Hand water or irrigate as needed.

Perennial Maintenance

- Deadhead flowers as needed.

- Prune to keep perennials looking neat and in proportion.

- Spray for insects or fungal diseases as needed.

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water.

- Adjust irrigation times as necessary.

- Check irrigation patterns to ensure irrigation is covering all areas.

- Mark irrigation heads prior to aeration

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Apply 28-5-12 50% Polyplus N + 3% Iron @ 1 bag per 14,000 sq. ft. per LESCO, if needed

  - Mow, weed eat, and edge lawn areas at a height of 2.5-3.5 inches depending on topography. Blow grass clippings from paved surfaces.

  - Pull Aerway aerifier, if needed

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Neighborhood Parks Landscape Maintenance

for September

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways when performing maintenance

- Report lighting and facility issues.

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Hand-pull weeds. (Pre-emergent is NOT safe to apply to annual beds.)

- Spray for insects as needed.

- Spray for fungal diseases as needed.

- Deadhead flowers as needed.

- Hand water or irrigate as needed.

Perennial Maintenance

- Deadhead flowers as needed.

- Prune to keep perennials looking neat and in proportion.

- Spray for insects or fungal diseases as needed.

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Check plant materials to ensure that they are receiving adequate water.

- Adjust irrigation times as needed.

- Check irrigation patterns to ensure irrigation is covering all areas.

- Mark irrigation heads prior to aeration

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 2.5-3.5 inches depending on topography. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - In preparation for overseeding, mow fescue 2 inches and aerate

  - Overseed fescue @ a rate of 8lbs. per 1000 sq. ft. Apply 18-24-12 starter fertilizer at the labeled rate, if present

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Neighborhood Parks Landscape Maintenance

for October

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways/parking lot when performing maintenance, remove leaves

- Report lighting and facility issues.

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

- Blow, rake, and remove leaves from beds.

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Plant winter annuals. Prepare beds by tilling 10-10-10 fertilizer into the top 6-8 inches of soil. Rake the beds smooth and top dress with Osmocote. Plant annuals and smooth soil between plants. Apply compost ½” to cover soil surface. Water plants in until soil is ‘soupy’.

- Hand water or irrigate as needed.

Perennial Maintenance

- Prune any unsightly perennials to neaten their appearance.

- Spray for insects or fungal diseases as needed.

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Start adjusting irrigation times back to prevent areas from being over-watered.

- Newly planted annuals and over-seeded lawns will need continued irrigation.

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 2”. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Planting

- Plant replacement trees and shrubs as needed.

Neighborhood Parks Landscape Maintenance

for November

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways/parking lot when performing maintenance, remove leaves

- Report lighting and facility issues.

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

- Blow, rake, and remove leaves from beds.

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Late November start planting spring-flowering bulbs. Plant tulips 6-inches deep on 4-inch spacing. Plant daffodils 6-inches deep on 6-inch spacing.

- Hand water or irrigate as needed.

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Begin turning off irrigation in preparation for freezing temps/winterizing

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 2”. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

Neighborhood Parks Landscape Maintenance

for December

General Park Maintenance

- Check restrooms daily for cleanliness

- Blow shelters, bridge for debris and spider webs

- Change trash bags in trash receptacles and litter pick as needed

- Check dog-waste bag supply when doing trash

- Blow walkways/parking lot when performing maintenance, remove leaves

- Report lighting and facility issues.

General Bed Maintenance

- Pull and weed eating any weeds over 6 inches tall, spray weeds as needed (pull weeds in direct contact with plant material, do not weed eat)

- Remove trash and cigarette butts from beds.

- Edge beds using gas-powered bed edger if needed

- Blow, rake, and remove leaves from beds.

Park Amenities Maintenance

- Address issues related to buildings and hardscapes. (Including but not limited to painting and replacement of rock in seating walls

- Paint/replace/repair benches, picnic tables, shelters, railings, trash receptacles as needed

- Pressure wash playground equipment as needed

Annual Maintenance

- Finish planting spring-flowering bulbs.

- Hand water or irrigate as needed.

Shrub Maintenance

- Light trimming as needed for aesthetic/safety

Tree Maintenance

- Limb-up trees to preserve views and prevent hazards.

- Check trees for hanging limbs that could present hazards.

- Ensure all invasives are kept off tree trucks and sprayed around the base periodically

Irrigation

- Should be turned off

- Hand water if needed

Lawn Maintenance

- Mowing maintenance should consist of the following: mowing, weed eating, edging, blowing pathway

- Warm-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 2”. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.

- Cool-season grasses

  - Mow, weed eat, and edge lawn areas at a height of 3 inches. Blow grass clippings from paved surfaces.

  - Monitor high traffic areas for excessive wear and rope off areas when necessary.